(Autonomous Batch)

SEMESTER END SUPPLEMENTARY EXAMINATIONS (AR21/23), SEP – 2026

| U.G. | ECE |  | Degree | Bachelor of Technology |  |
| --- | --- | --- | --- | --- | --- |
| Academic Year | 2026-2027 |  | Sem. | 4th |  |
| Course Code | 21EC402/23EC402 | Course Title |  |  |  |
|  |  | Analog Electronic Circuits |  |  |  |
| Duration | 3 Hours | Maximum Marks |  |  | 70 (SEVENTY) |

SECTION-I

7 x 2 = 14 Marks

1.

| No. | Questions (a to g) | RBT Level | COs |
| --- | --- | --- | --- |
| a | Reproduce the block diagram of current series feedback amplifier. | Remember | 1 |
| b | Show that band width improved with negative feedback. | Remember | 2 |
| c | Classify power Amplifiers. | Remember | 2 |
| d | What is stagger tuned amplifier? | Remember | 3 |
| e | What is the condition of a RC high pass circuit to acts as differentiator? | Remember | 4 |
| f | Draw a circuit diagram of a clamper circuit. | Remember | 4 |
| g | If R1=10KΩ, R2=5KΩ and C1= C2=0.1µF, find the frequency of the astable output. | Remember | 6 |

SECTION-II

4 x 14 = 56 Marks

| No. |  | Questions (2 to 9) |  |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) |  | Explain the principle of negative feedback in amplifiers. Show quantitatively the effect of negative feedback on (i) Gain (ii) Stability (iii) Noise (iv) Distortion. | Understand | 1 | 8M |
|  |  | (b) |  | Calculate the closed loop gain ACL for a voltage series negative feedback amplifier if its open loop gain Av and feedback factor β are 103 and 0.01 respectively. Calculate ACL if Av increases by 40%. | Apply | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 3 |  | (a) |  | Derive the expression frequency of oscillation and condition for sustained oscillations of a Colpitts oscillator. | Apply | 1 | 8M |
|  |  | (b) |  | Derive the basic conditions for oscillations and classify oscillators. | Apply | 2 | 6M |
| 4 |  | (a) |  | Explain the operation of class A push-pull power amplifier. | Understand | 3 | 8M |
|  |  | (b) |  | With the help of a suitable circuit diagram, show that the maximum conversion efficiency of a class B power amplifier is 78.5% | Understand | 2 | 6M |
| OR |  |  |  |  |  |  |  |
| 5 |  | (a) | Write a short note on single tuned amplifier. |  | Understand | 3 | 8M |
|  |  | (b) | Draw the circuit diagram of a double tuned amplifier and explain its gain frequency response. |  | Understand | 2 | 6M |
| 6 |  | (a) | A square wave whose peak to peak amplitude is 2 V extends ± 1 V with respect to ground. The duration of the positive section is 0.1 s and that of the negative section is 0.2s. if this waveform is impressed upon an RC integrating circuit whose time constant is 0.2s, what are the steady-state maximum and minimum values of the output waveform? |  | Apply | 4 | 8M |
|  |  | (b) | Explain the response of High-pass RC circuit for step input. |  | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 7 |  | (a) |  | State and explain clamping circuit theorem. | Understand | 4 | 8M |
|  |  | (b) |  | Draw the basic circuit of diode clipper and explain its operation with the help of transfer characteristics. | Understand | 5 | 6M |
| 8 |  | (a) |  | Write and prove the relation between slope error(es), displacement error(ed) and transmission error(et) with reference to a sweep circuit. | Understand | 6 | 8M |
|  |  | (b) |  | Draw and explain the transistorized Miller sweep circuit. | Understand | 5 | 6M |
| OR |  |  |  |  |  |  |  |
| 9 | (a) |  |  | With suitable waveforms, explain the function of a Monostable multivibrator. | Understand | 6 | 8M |
|  | (b) |  |  | Explain the operation of Astable multivibrator with suitable wave forms. | Understand | 5 | 6M |

***********