(Autonomous Batch)

SEMESTER END SUPPLEMENTARY EXAMINATIONS (AR23), SEPT. – 2026

| Degree | B.Tech. |  | Program | EEE |
| --- | --- | --- | --- | --- |
| Academic Year | 2026-2027 |  | Semester | 4th |
| Course Code | 21EE401/23EE401 | Course Title |  | AC Machines |
| Duration | 3 Hours | Max. Marks |  | 70 (Seventy) |

SECTION-I

7 x 2 = 14 Marks

1.

| No. | Questions (a to f) | RBT Level | COs |
| --- | --- | --- | --- |
| a | list the parts of three phase induction motor. | Remember | CO1 |
| b | Define Slip. | Remember | CO2 |
| c | List out any four starting methods of three phase induction motor. | Remember | CO3 |
| d | What is Double field revolving theory? | Remember | CO3 |
| e | Define basic working principle of an alternator. | Remember | CO4 |
| f | Write the E.M.F equation of an alternator. | Remember | CO4 |
| g | Draw the phasor diagram foe synchronous motor when a) it is under excited b) it is over excited | Understand | CO6 |

SECTION-II

4 x 14 = 56 Marks

| No. |  | Questions (2 to 9) |  |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 |  | (a) |  | Derive the expressions for starting and maximum torques of three phase induction motor. | Understand | CO1 | 8M |
|  |  | (b) |  | A 6-pole, 50Hz, three phase induction motor running on full-load with 4% slip develops a torque of 149.3N-m at its rim. The friction and windage losses are 200W and the stator copper loss and iron losses are 1620W. Calculate i) output power ii) rotor copper loss iii) efficiency at full load | Apply | CO2 | 6M |
| OR |  |  |  |  |  |  |  |
| 3 |  | (a) |  | Write the procedure to construct a circle diagram with a neat diagram | Understand | CO1 | 8M |
|  |  | (b) |  | Explain the production of R.M.F in three phase induction motor. | Understand | CO2 | 6M |
| 4 |  | (a) |  | Explain the rotor resistance control and voltage injection control methods of a three phase induction motor. | Understand | CO3 | 8M |
|  |  | (b) |  | Explain the working of universal motor and write its applications. | Understand | CO2 | 6M |
| OR |  |  |  |  |  |  |  |
| 5 |  | (a) | Explain briefly about the auto-transformer starting of three phase induction motor. |  | Understand | CO3 | 8M |
|  |  | (b) | Explain the working of BLDC motor and write its applications. |  | Understand | CO2 | 6M |
| 6 |  | (a) | Explain the constructional details of Synchronous generator. |  | Understand | CO4 | 8M |
|  |  | (b) | Explain about the synchronization of alternators with the infinite bus. |  | Understand | CO5 | 6M |
| OR |  |  |  |  |  |  |  |
| 7 |  | (a) |  | Derive the expression for synchronizing power in an alternator. | Understand | CO4 | 8M |
|  |  | (b) |  | A given 3-MVA, 50-Hz, 11KV, three phase star-connected alternator when supplying 100A at zero p.f leading has a line-line voltage of 12370V;when the load is removed the terminal voltage falls down to 11000V.Predict the regulation of alternator when supplying full load ata0.8 p.f lag. Assume effective resistance of 0.4 ohm per phase. | Apply | CO5 | 6M |
| 8 |  | (a) |  | Illustrate the working principle of synchronous motor with neat diagram. | Understand | CO6 | 8M |
|  |  | (b) |  | Compare induction motor and synchronous motor. | Understand | CO5 | 6M |
| OR |  |  |  |  |  |  |  |
| 9 | (a) |  |  | Explain the operation of synchronous condenser. | Understand | CO6 | 8M |
|  | (b) |  |  | Define Hunting. Determine how it is suppressed in a synchronous motor? | Understand | CO5 | 6M |

***********