(Autonomous Batch)

SEMESTER END SUPPLEMENTARY EXAMINATIONS (AR21/23), SEPT. – 2026

| Degree | B.Tech. |  | Program | ME |
| --- | --- | --- | --- | --- |
| Academic Year | 2026-2027 |  | Semester | 4th |
| Course Code | 21ME404/23ME404 | Course Title |  | Mechanics of Solids |
| Duration | 3 Hours | Max. Marks |  | 70 (Seventy) |

SECTION-I

7 x 2 = 14 Marks

1.

| No. | Questions (a to g) | RBT Level | COs |
| --- | --- | --- | --- |
| a | State Hooke’s law. | Remember | CO1 |
| b | Define flexural rigidity. | Remember | CO1 |
| c | A solid shaft transmits 150kW power at 180r.p.m. Compute the torque transmitted by a solid shaft. | Remember | CO3 |
| d | What is pure torsion? | Remember | CO3 |
| e | What is meant by Hoop stress and longitudinal stress? | Remember | CO4 |
| f | Draw the shear stress distribution diagram for 'L' type section. | Remember | CO4 |
| g | What do you understand by compound cylinders subjected to internal fluid pressure? | Remember | CO6 |

Section II (Long Answer Questions)

(Answer all Questions)

4 x 14 = 56 Marks

| No. | Questions (2 to 9) |  | RBT Level | COs | Marks |
| --- | --- | --- | --- | --- | --- |
| 2 | (a) | Explain the stress strain curve for mild steel with neat diagram. | Understand | CO1 | 8M |
|  | (b) | A composite tube consists of a steel tube 150mm internal diameter and 10mm thickness and an outer brass tube 170mm internal diameter and 10mm thickness. The two tubes are of the 150mm length. The compound tube carries an axial load of 1000kN. Compute (i) stress in the tubes and (ii) load carried by each tube Take Es=2x105N/mm2 and Eb=1x105N/mm2. | Apply | CO2 | 6M |
|  | OR |  |  |  |  |
| 3 | (a) | Draw the shear force and bending moment diagrams for the simply supported beam as shown in figure. | Apply | CO1 | 8M |
|  | (b) | Draw the shear force and bending moment diagrams for the simply supported beam subjected to point load at the centre. | Apply | CO2 | 6M |
| 4 | (a) | Derive the bending equation and state the assumptions. | Understand | CO3 | 8M |
|  | (b) | An I – section beam shown in figure, is simply supported over a span of 12m. If the maximum permissible bending stress is 80N/mm2, what concentrated load can be carried at a distance of 4m from one support? | Apply | CO2 | 6M |
|  | OR |  |  |  |  |
| 5 | (a) | Prove that the maximum shear stress in a rectangular section of a beam is 1.5 times the average shear stress. | Apply | CO3 | 8M |
|  | (b) | A circular beam of 105mm diameter is subjected to a shear force of 5000N. Estimate the value of maximum shear stress and sketch the variation of the shear stress along the depth of the beam. | Apply | CO2 | 6M |
| 6 | (a) | Derive the pure torsion equation for shaft subject to torque. | Understand | CO4 | 8M |
|  | (b) | Write the assumptions of the pure torsion equation and explain the section modulus for rectangle section. | Understand | CO5 | 6M |
|  | OR |  |  |  |  |
| 7 | (a) | A cantilever of length 2m and cross section 100mmx200mm is loaded by 20kW at the free end. Compute the maximum slope and deflection at the free end. Take E=0.2x105N/mm2. | Apply | CO4 | 8M |
|  | (b) | A beam of length 6m is simply supported at its ends over a and carries two concentrated loads of 48kN and 40kN at a distance of 1m and 3m respectively from the left support. Analyze the deflection under the load. Take E=2x105N/mm2and I=85x106mm4. | Apply | CO5 | 6M |
| 8 | (a) | Derive an expression for circumferential stress for a thin cylinder subjected to an internal pressure. | Understand | CO6 | 8M |
|  | (b) | A cylinder of internal diameter 2.5m and of thickness 50mm contains a gas. If the tensile stress in the material is not to exceed 80N/mm2, analyze the internal pressure of the gas. | Apply | CO5 | 6M |
|  | OR |  |  |  |  |
| 9 | (a) | Evaluate the maximum hoop stress across the section of a pipe of external diameter 600mm and internal diameter 440mm, when the pipe is subjected to an internal fluid pressure of 9N/mm2. | Apply | CO6 | 8M |
|  | (b) | Sketch the radial pressure distribution and hoop stress distribution across the section for the above problem. | Apply | CO5 | 6M |

*******